1.General The ship is to be buit in acordance with the ABS approvaled plans and the “steel barge constrution and classification regulation” and its later amendment (1991) of ABS. It is to be inspected and surveyed by ABS class and is to be registed as an “unmanned Deck Cargo Barge”. 2.Principle particulars Length overall 270Ft Beam moulded 70Ft Depth moulded 16Ft Deck average loading 5ton / m2 Deadweight approx. 5000 tons 3.Line and Layout The main hull is to be divided by eight(8)transverse W.T bulkhead and two(2) longiyudinal W.T bulkheads into twenty-seven(27) compartments, one flush manhote hatch and one ladder in every void space. 4.Main material specification Deck 12mm Shell plate 12mm Longitudinal BHD 8mm Transverse BHD 8mm Bottom longitudinal 150x90x9 First futtock 160x100x12 Shell longitudinal 150x90x9 Deck longitudinal 125x80x8 Sideboard high 10’ 5.Deck fittings Mooring Bollards 8nos Mooring hawses 2nos 800kg hawer anchor 1nos 4t manual mooring winch 1nos Navingation light brackets 3nos Battery boxes 6nos 600 x 400flush manhole covers 27nos 6.Paint China-made Paint.